# Automotive OEM / Tier-1 Master QMS Architecture & Product Lifecycle Framework

Document Identifier: COMPANYID-QMS-MAST-2026-V1
Target Organization: Automotive OEM & Electronics Tier-1 Integrated Systems Manufacturing
Standard Alignment: IATF 16949:2016, ISO 9001:2015, ISO 27001:2022, ISO 14001:2015, ISO 45001:2018, APQP Manual 2nd Ed.Operational Instruction: This master document establishes the mandatory baseline requirements for product development, quality management, facility compliance, and cross-functional execution. Program managers, process owners, and audit leads are authorized to edit, expand, or adapt specific stage parameters, gateway entry/exit criteria, and department metrics to meet OEM Customer Specific Requirements (CSRs) or project-specific scopes.

# 1. 37-Stage Automotive Product Lifecycle Flow (HW_01 to HW_37)

The following lifecycle framework maps stages HW_01 through HW_37 across Advanced Product Quality Planning (APQP) Phase Gates 1 to 5 and IATF 16949:2016 Clause 8.3 (Design and Development of Products and Services).+------------------------------------------------------------------------------------------------------+

|                                    APQP PHASE GATE STRUCTURE                                         |

|---|

| APQP Gate 1: Plan & Define         | APQP Gate 2: Design & Development  | APQP Gate 3: Process Dev   |

| (HW_01 to HW_07)                   | (HW_08 to HW_16)                   | (HW_17 to HW_24)           |

+------------------------------------+------------------------------------+----------------------------+

| APQP Gate 4: Validation & PPAP     | APQP Gate 5: Ramp-Up & Feedback                                |

| (HW_25 to HW_31)                   | (HW_32 to HW_37)                                               |

+------------------------------------+-----------------------------------------------------------------+

## APQP Phase Gate 1: Planning & Program Definition (Clause 8.3.2)

### HW_01: Market Requirement & RFQ Intake

- Lead Department: Sales & Business Development

- Interfacing Departments: Systems Engineering, Embedded HW, SCM, Operations

- Input Prerequisites: OEM Request for Quote (RFQ), Statement of Work (SOW), Customer Specific Requirements (CSRs)

- Output Artifacts: Market Requirements Document (MRD), Project Charter, Feasibility Sign-off Form

- IATF Audit Evidence: Clause 8.2.2 & 8.2.3 Review Records, Feasibility Analysis Report

- Feedback Loop: Feasibility findings feed back into Sales for commercial quote revisions.

### HW_02: Customer Technical Requirement (CTR) Freeze

- Lead Department: Systems & Systems Engineering

- Interfacing Departments: Embedded HW, Embedded SW, Mechanical Engineering, Sales

- Input Prerequisites: OEM Technical Specification, Vehicle Architecture Baseline

- Output Artifacts: System Requirements Specification (SRS), Customer Traceability Matrix (CTM)

- IATF Audit Evidence: Clause 8.3.3.1 Engineering Input Verification, System SRS Sign-off

- Feedback Loop: Unclear or conflicting requirements trigger formal Clarification Requests to the OEM.

### HW_03: Business Case & Sourcing Feasibility

- Lead Department: Operations & Finance

- Interfacing Departments: Purchasing, Sales, Manufacturing Engineering

- Input Prerequisites: Bill of Materials (BOM) cost estimates, tooling CAPEX estimates, volume forecasts

- Output Artifacts: Finalized Business Case Model, IRR/NPV Calculation, Target Cost Allocation

- IATF Audit Evidence: Gate 1 Financial Assessment Sign-off, Sourcing Board Minutes

- Feedback Loop: Target cost breaches trigger redesign or component renegotiation cycles.

### HW_04: High-Level System & HW Architecture Definition

- Lead Department: Embedded Hardware Design

- Interfacing Departments: Embedded SW, Mechanical Engineering, Thermal & Reliability

- Input Prerequisites: System SRS, Vehicle Voltage & Bus Standards (CAN/LIN/Ethernet)

- Output Artifacts: Hardware Architecture Document (HAD), System Block Diagram, Power Budget Model

- IATF Audit Evidence: Architecture Design Review Minutes, Interface Control Document (ICD)

- Feedback Loop: Thermal or power constraints loop back to modify SRS power allocations.

### HW_05: Initial Safety & Cybersecurity Hazard Analysis

- Lead Department: Quality & Functional Safety (FuSa)

- Interfacing Departments: Systems Engineering, Embedded HW, Embedded SW

- Input Prerequisites: System Operational Architecture, Vehicle Hazard Inputs

- Output Artifacts: Item Definition, Hazard Analysis and Risk Assessment (HARA), Threat Analysis and Risk Assessment (TARA)

- IATF Audit Evidence: ISO 26262 / ISO 21434 Safety & Cybersecurity Concept Audit Dossier

- Feedback Loop: Safety Goals (ASIL levels) force design adjustments to HW Architecture (HW_04).

### HW_06: Preliminary Bill of Materials (p-BOM) & Sourcing Strategy

- Lead Department: Purchasing / Supply Chain Management

- Interfacing Departments: Embedded HW, Component Quality, Component Engineering

- Input Prerequisites: Hardware Schematic Architecture, Approved Vendor List (AVL)

- Output Artifacts: Preliminary BOM (p-BOM), Long Lead Time Component Analysis, Single Source Risk Matrix

- IATF Audit Evidence: Clause 8.4 Supplier Selection Records, SCM Feasibility Assessment

- Feedback Loop: Obsolescence or lead-time issues trigger immediate component substitution in p-BOM.

### HW_07: APQP Phase Gate 1 Approval & Program Approval

- Lead Department: Program Management Office (PMO)

- Interfacing Departments: Executive Leadership, All Department Heads

- Input Prerequisites: Completed HW_01 through HW_06 deliverables and gate review audit checklist

- Output Artifacts: Gate 1 Sign-Off Certificate, Program Master Schedule (Gantt), Gate Baseline Approval

- IATF Audit Evidence: Clause 8.3.2 Planning Gate Review Minutes and Sign-off Sheets

- Feedback Loop: Failed gate items delay program execution until Open Action Items are resolved.

## APQP Phase Gate 2: Product Design & Development (Clause 8.3.3 & 8.3.4)

### HW_08: Detailed Hardware Schematic Capture

- Lead Department: Embedded Hardware Design

- Interfacing Departments: Component Engineering, Functional Safety, SCM

- Input Prerequisites: Hardware Architecture (HW_04), Final Component Selection Data Sheets

- Output Artifacts: Schematic Capture Schematics (.DSN/.SchDoc), Electronic BOM (eBOM v1.0)

- IATF Audit Evidence: Design Review Checklist, DRC (Design Rule Check) Clean Log

- Feedback Loop: Schematic errors caught in peer review trigger internal schematic revisions.

### HW_09: Hardware DFMEA Execution

- Lead Department: Quality Assurance & Hardware Engineering

- Interfacing Departments: Systems, Mechanical Engineering, Manufacturing Engineering

- Input Prerequisites: Hardware Schematics, Boundary Diagrams, Interface Matrices, Historical Lesson Learned

- Output Artifacts: Hardware DFMEA Document (AIAG-VDA aligned), Action Priority (AP) Mitigation Plan

- IATF Audit Evidence: Clause 8.3.5.1 DFMEA Report, High AP Action Tracking Log

- Feedback Loop: High AP risks drive circuit redesign, component de-rating, or diagnostic additions.

### HW_10: High-Speed PCB Layout & Routing

- Lead Department: Hardware Design (PCB Layout Team)

- Interfacing Departments: Mechanical Engineering, Signal Integrity (SI/EMC), Electronics Manufacturing

- Input Prerequisites: Schematic Nets, Mechanical Enclosure Keep-out Zones, Stack-up Definition

- Output Artifacts: PCB Artwork, Gerber Files, IPC-2581/ODB++ CAD Package, Stack-up Report

- IATF Audit Evidence: PCB DFM/DFA Sign-off, Signal/Power Integrity Analysis Report

- Feedback Loop: DFM violations flagged by manufacturing force layout modifications prior to tape-out.

### HW_11: Mechanical Enclosure & Thermal CAD Design

- Lead Department: Mechanical Engineering

- Interfacing Departments: Embedded HW, Thermal Engineering, Tooling Engineering

- Input Prerequisites: PCB 3D STEP Model, Vehicle Packaging Space, IP Rating Requirements

- Output Artifacts: 3D CAD Enclosure Model, 2D Mechanical Tooling Drawings, Structural FEA Report

- IATF Audit Evidence: Mechanical CAD Baseline Sign-off, Thermal Simulation Report

- Feedback Loop: Thermal hotspots identified in simulation require heat-sink or enclosure design changes.

### HW_12: Design for Manufacturability (DFM) & Assembly (DFA) Review

- Lead Department: Electronics & Mechanical Manufacturing

- Interfacing Departments: Hardware Design, Mechanical Design, Quality Assurance

- Input Prerequisites: PCB Layout Gerber, 3D Mechanical CAD, Panelization Specs

- Output Artifacts: DFM/DFA Audit Report, Manufacturing Clearance Sign-off

- IATF Audit Evidence: DFM/DFA Gate Review Checklist, Engineering Change Request (ECR) if issues found

- Feedback Loop: Panel size or component clearance issues drive PCB/CAD layout updates.

### HW_13: Prototype Hardware Component Procurement (A-Sample)

- Lead Department: Purchasing / SCM

- Interfacing Departments: Embedded HW, Component Quality, Prototype Workshop

- Input Prerequisites: eBOM v1.0, Gerber Package, Stencil Specs, Fast-track Purchase Requisitions

- Output Artifacts: Prototype Component Kits, Quick-turn PCB Fabrication Order, Prototype Tooling Orders

- IATF Audit Evidence: Clause 8.4 Component Traceability, Receiving Inspection Reports

- Feedback Loop: Component availability delays force fast-track engineering substitutions.

### HW_14: A-Sample Board Assembly & Bench Bringing-Up

- Lead Department: Prototype & Engineering Lab

- Interfacing Departments: Embedded HW, Embedded SW, Quality Assurance

- Input Prerequisites: Naked PCBs, Component Kits, Stencils, Assembly Drawings, Hardware Bringing-up Guide

- Output Artifacts: Assembled A-Sample PCBA, Initial Smoke Test Report, Hardware Bring-up Checklist

- IATF Audit Evidence: Prototype Assembly Records, HW Modification Log (Rework Trackers)

- Feedback Loop: Hardware bugs found during bring-up result in ECO/ECN fixes for B-Sample layout.

### HW_15: Hardware Design Verification Plan (DVP) Creation

- Lead Department: Test & Validation Engineering

- Interfacing Departments: Embedded HW, Systems Engineering, Quality Assurance

- Input Prerequisites: Customer Specifications, ISO 16750, OEM Environmental Specs

- Output Artifacts: Hardware DVP matrix, Test Case Descriptions, Equipment Calibration Specs

- IATF Audit Evidence: Clause 8.3.4.2 Verification Plan Document, Customer DVP Approval

- Feedback Loop: Test case gaps identified by the OEM force amendments to the DVP document.

### HW_16: APQP Phase Gate 2 Approval & Design Freeze

- Lead Department: Program Management Office (PMO)

- Interfacing Departments: All Engineering & Operations Heads

- Input Prerequisites: A-Sample Bring-up sign-off, DFMEA completion, DVP release

- Output Artifacts: Gate 2 Sign-Off Certificate, A-Sample Validation Report, Engineering Freeze Baseline

- IATF Audit Evidence: Clause 8.3.4 Gate Review Minutes, Design Freeze Sign-off

- Feedback Loop: Unresolved hardware issues block Gate 2 passage and delay prototype builds.

## APQP Phase Gate 3: Process Design & Development (Clause 8.3.4 & 8.3.5)

### HW_17: Manufacturing Process Flow & Layout Definition

- Lead Department: Electronics & Mechanical Manufacturing Engineering

- Interfacing Departments: Operations, Quality Assurance, EHS & Facilities

- Input Prerequisites: Frozen A-Sample CAD/Schematics, Production Volume Forecasts

- Output Artifacts: Manufacturing Process Flow Diagram (PFD), Plant Line Layout Diagram, Ergonomic Study

- IATF Audit Evidence: Process Flow Diagram Document, Floor Layout Optimization Sign-off

- Feedback Loop: Space or line capacity constraints require optimization of line steps or takt times.

### HW_18: Process Failure Mode & Effects Analysis (PFMEA)

- Lead Department: Manufacturing Quality Engineering

- Interfacing Departments: Manufacturing Engineering, Hardware Design, Operations, Service

- Input Prerequisites: Process Flow Diagram, DFMEA, Historical Lesson Learned DB (e.g., Wire Sheath Gland NC)

- Output Artifacts: PFMEA Document (AIAG-VDA Aligned), Action Priority Mitigation Plan

- IATF Audit Evidence: Clause 8.3.5.2 PFMEA Report, Mistake-Proofing (Poka-Yoke) Implementation Log

- Feedback Loop: High AP process risks drive mandatory Poka-Yoke fixture installation on the production line.

### HW_19: Pre-Launch Control Plan & Work Instructions (SOP)

- Lead Department: Manufacturing Quality Engineering

- Interfacing Departments: Production, Hardware Design, Quality Assurance

- Input Prerequisites: PFMEA, Process Flow Diagram, Customer Specific Requirements

- Output Artifacts: Pre-Launch Control Plan, Standard Operating Procedures (SOPs), Visual Inspection Aids

- IATF Audit Evidence: Clause 8.5.1 Control Plan Document, Approved Work Station Instructions

- Feedback Loop: Quality escapes during pilot production force Control Plan updates and tighter inspection frequencies.

### HW_20: Production Tooling, Jigs, Fixtures & Stencils Release

- Lead Department: Tooling & Equipment Engineering

- Interfacing Departments: Mechanical Engineering, Manufacturing, Quality Assurance

- Input Prerequisites: Frozen 3D CAD Enclosure, PCB Panel Layout, Test Point Locations

- Output Artifacts: Reflow Stencils, ICT/FCT Test Fixtures, Casing Assembly Jigs, Automated Screw Driver Units

- IATF Audit Evidence: Tooling Verification Reports, Calibration and Acceptance Certificates

- Feedback Loop: Fixture alignment or contact-pin failures force fixture rework and adjustment.

### HW_21: Measurement Systems Analysis (MSA) Plan Execution

- Lead Department: Quality Assurance (Metrology)

- Interfacing Departments: Manufacturing Engineering, Validation Lab

- Input Prerequisites: Measurement Gauges, Automated Optical Inspection (AOI), Callipers, FCT Testers

- Output Artifacts: Gage R&R Reports, Linearity & Bias Analysis, Attribute Agreement Analysis (AAA)

- IATF Audit Evidence: Clause 7.1.5.1.1 MSA Studies (Gage R&R < 10% acceptable, 10-30% conditional)

- Feedback Loop: Gage R&R > 30% forces gauge re-calibration, fixture redesign, or operator retraining.

### HW_22: B-Sample Prototype Build (Pilot Line Assembly)

- Lead Department: Electronics & Mechanical Manufacturing

- Interfacing Departments: Purchasing, Quality Assurance, Hardware Engineering

- Input Prerequisites: Tooling & Fixtures, B-Sample Components, SOPs, Control Plan

- Output Artifacts: B-Sample Units, Pilot Run Build Report, First Pass Yield (FPY) Metrics

- IATF Audit Evidence: Pilot Build Traveler Records, Traceability Log

- Feedback Loop: Assembly bottlenecks or component fit issues generate immediate ECRs to HW/Mech design.

### HW_23: B-Sample DVP&R Functional & Environmental Testing

- Lead Department: Test & Validation Engineering

- Interfacing Departments: Quality Assurance, Hardware Engineering, OEM Quality

- Input Prerequisites: B-Sample Hardware, Approved DVP Matrix, Calibrated Environmental Chambers

- Output Artifacts: Thermal Cycling Reports, Vibration/Drop Test Reports, EMC/EMI Pre-compliance Results

- IATF Audit Evidence: DVP&R Test Reports signed by NABL/ISO 17025 accredited lab manager

- Feedback Loop: Test failures (e.g., EMC emissions overshoot) force filter design changes or shield additions.

### HW_24: APQP Phase Gate 3 Approval & Process Freeze

- Lead Department: Program Management Office (PMO)

- Interfacing Departments: Operations, Quality, Manufacturing, Engineering

- Input Prerequisites: Complete PFMEA, Pre-Launch Control Plan, B-Sample Test Verification

- Output Artifacts: Gate 3 Sign-Off Certificate, Process Freeze Document

- IATF Audit Evidence: Clause 8.3.4 Gate Review Minutes, Process Baseline Acceptance

- Feedback Loop: Unfinished PFMEA mitigations block entry into Gate 4.

## APQP Phase Gate 4: Product & Process Validation (Clause 8.3.4, 8.3.5 & 8.5.1)

### HW_25: Pilot Production Run / Run-at-Rate (C-Sample Build)

- Lead Department: Operations & Manufacturing Engineering

- Interfacing Departments: Quality Assurance, SCM, Program Management

- Input Prerequisites: Production Tooling, Final Plant Line Setup, Validated SOPs

- Output Artifacts: C-Sample Units, Run-at-Rate Performance Report, Capacity Verification Report

- IATF Audit Evidence: Clause 8.5.1.4 Run-at-Rate Audit Minutes, Line Speed & OEE Metrics

- Feedback Loop: Failure to achieve target OEE or cycle times delays mass production launch.

### HW_26: Statistical Process Control (SPC) Capability Evaluation

- Lead Department: Manufacturing Quality Assurance

- Interfacing Departments: Manufacturing Engineering, Operations

- Input Prerequisites: Dimensional inspection data, Critical-to-Quality (CTQ) parameters from Run-at-Rate

- Output Artifacts: Process Capability Report (Cpk / Ppk analysis for CTQ parameters)

- IATF Audit Evidence: Clause 9.1.1.2 SPC Records (Target Ppk ≥ 1.67, Cpk ≥ 1.33)

- Feedback Loop: Ppk < 1.67 mandates 100% sorting/inspection and 8D root-cause investigation.

### HW_27: Full Environmental & EMC Product Homologation

- Lead Department: Validation & Compliance Engineering

- Interfacing Departments: External Accredited Test Agency (e.g., ARAI, ICAT, TÜV), Quality

- Input Prerequisites: C-Sample Production Units, Final Vehicle Integration Harness

- Output Artifacts: Homologation Compliance Certificates (CE, E-Mark, CISPR 25, ISO 11452)

- IATF Audit Evidence: Official Homologation Test Certificates, Regulatory Compliance Dossier

- Feedback Loop: Failure in regulatory test cancels PPAP submission and triggers re-design.

### HW_28: Production Part Approval Process (PPAP) Package Assembly

- Lead Department: Quality Assurance

- Interfacing Departments: All Departments (Engineering, SCM, Operations, Validation)

- Input Prerequisites: 18 standard PPAP elements (Design Records, DFMEA, PFMEA, Control Plan, MSA, Cpk, PSW, etc.)

- Output Artifacts: Complete PPAP Binder (Level 1-5 as per customer demand), Part Submission Warrant (PSW)

- IATF Audit Evidence: Clause 8.3.5.3 / 8.5.6 PPAP Submission Package

- Feedback Loop: Rejection of PPAP by OEM requires immediate containment and corrective action plan.

### HW_29: Production Control Plan (PCP) & Final Line Handover

- Lead Department: Quality Assurance & Operations

- Interfacing Departments: Production, Plant Quality, Maintenance

- Input Prerequisites: Approved PPAP, Finalized SPC, Poka-Yoke validations

- Output Artifacts: Production Control Plan (PCP v1.0), Line Handover Certificate

- IATF Audit Evidence: Clause 8.5.1 Operational Production Control Plan

- Feedback Loop: Line operational escapes update PCP inspection steps.

### HW_30: Customer PPAP Approval & Part Submission Warrant (PSW) Sign-off

- Lead Department: Sales & Quality Assurance

- Interfacing Departments: Program Management, OEM Quality

- Input Prerequisites: Submitted PPAP Dossier and Sample Parts

- Output Artifacts: Approved Customer PSW Sign-off, Customer Purchase Approval

- IATF Audit Evidence: Customer PSW Approval Document

- Feedback Loop: Conditional PSW approvals trigger containment actions until full approval.

### HW_31: APQP Phase Gate 4 Approval & Production Readiness

- Lead Department: Program Management Office (PMO)

- Interfacing Departments: Executive Committee, Operations, SCM, Quality

- Input Prerequisites: Signed PSW, Capacity Verification, Mass Production Supply Contracts

- Output Artifacts: Gate 4 Approval Certificate, Mass Production Authorization Order

- IATF Audit Evidence: Clause 8.3.4 Gate Review Minutes, Gate 4 Clearance Sign-off

- Feedback Loop: Open quality actions hold product dispatch authority.

## APQP Phase Gate 5: Ramp-Up, Operations & Feedback (Clause 8.5, 9.1, 10.2)

### HW_32: Mass Production SOP & Ramp-Up Execution

- Lead Department: Operations & Manufacturing

- Interfacing Departments: SCM, Quality, Plant Maintenance

- Input Prerequisites: Mass Production Authorization, Approved PSW, Supply Chain Baseline

- Output Artifacts: Daily Production Schedules, Actual vs Planned Yield Reports

- IATF Audit Evidence: Clause 8.5.1 Production Scheduling & Execution Logs

- Feedback Loop: Production line yield drops trigger immediate floor stop and line containment.

### HW_33: Early Production Containment Plan (EPCP / Safe Launch)

- Lead Department: Quality Assurance (Plant Quality)

- Interfacing Departments: Production, SCM, Hardware Engineering

- Input Prerequisites: SOP Start, Customer Ramp-up Schedule

- Output Artifacts: EPCP Checklist, Safe Launch Inspection Log (100% redundant checking)

- IATF Audit Evidence: Safe Launch Containment Records (Typically first 90 days or 5,000 units)

- Feedback Loop: Zero-defect milestone achievement allows exit from Safe Launch back to standard PCP.

### HW_34: Post-Launch Lessons Learned & Knowledge Management

- Lead Department: Program Management & Quality

- Interfacing Departments: All Engineering & Operations Teams

- Input Prerequisites: Program Ramp-Up Metrics, Gate Escalation Logs, Initial Quality Data

- Output Artifacts: Program Lessons Learned Document, Updated Design Guidelines, Updated PFMEA Library

- IATF Audit Evidence: Clause 7.1.6 Organizational Knowledge Records, Lessons Learned DB Entries

- Feedback Loop: Lessons learned directly update DFMEA/PFMEA template libraries for future RFQs.

### HW_35: Warranty & Field Return 8D Problem Solving

- Lead Department: Service & Quality Engineering

- Interfacing Departments: Hardware Design, firmware design, Software Design, Manufacturing Quality

- Input Prerequisites: Customer Field Claims, Returned Non-conforming Hardware Units

- Output Artifacts: Formal 8D Corrective Action Reports, Teardown Reports, 5-Why Cause Analysis

- IATF Audit Evidence: Clause 10.2.3 Problem Solving (8D) Reports, Root Cause Validation Evidence

- Feedback Loop: Root cause identification generates ECNs for design or tooling improvements.

### HW_36: Continuous Improvement (Kaizen) & VAVE Implementation

- Lead Department: Manufacturing Engineering & SCM

- Interfacing Departments: Hardware Engineering, Operations, Quality

- Input Prerequisites: Running Production Data, Field Performance Data, Cost Reduction Targets

- Output Artifacts: Value Analysis / Value Engineering (VAVE) Proposals, Kaizen Projects Log

- IATF Audit Evidence: Clause 10.3 Continuous Improvement Records

- Feedback Loop: Validated VAVE proposals require re-entering the APQP cycle at Gate 2/3 (ECN path).

### HW_37: Engineering Change Management (ECN / ECO Loop)

- Lead Department: Hardware Engineering & Configuration Management

- Interfacing Departments: All Departments

- Input Prerequisites: Engineering Change Request (ECR), Field 8D, VAVE Proposal, Component EOL

- Output Artifacts: Engineering Change Notice (ECN), Revised BOM, Updated Drawings, Re-PPAP Matrix

- IATF Audit Evidence: Clause 8.5.6 Control of Changes Records, ECN Sign-off Matrix

- Feedback Loop: ECN execution updates lifecycle baseline configurations across PLM and ERP systems.

# 2. Cross-Departmental Communication & Document Dependency Matrix

+-------------------------------------------------------------------------------------------------------------------------+

|                                    CROSS-DEPARTMENTAL DEPENDENCY & FEEDBACK LOOPS                                       |

+----------------------+--------------------+---------------------+-------------------------------------------------------+

| Trigger Event        | Source Dept        | Target Depts        | Key Documents & Action Required                       |

+----------------------+--------------------+---------------------+-------------------------------------------------------+

| 1. Requirements Sync | Sales / Systems    | HW / SW / Mech      | CTR, SRS, CTM -> Traceability matrix update           |

| 2. Mech Validation   | Mech Engineering   | HW Design / Mfg     | 3D STEP, Stack-up -> DFM/DFA Sign-off                 |

| 3. DVR Testing Defect| Validation Lab     | HW Design / Quality | Test Defect Report -> 8D Analysis & ECR Generation    |

| 4. Post-Release ECO  | Plant Quality / HW | SCM / Mfg / Ops     | ECN Package -> BOM Update, Tooling Adjust, Re-PPAP    |

+----------------------+--------------------+---------------------+-------------------------------------------------------+

## Detailed Bi-Directional Feedback Loops

### Loop 1: Requirements Sync Loop (Sales <-> Systems <-> R&D)

- Mechanism: When OEM issues a requirement revision, Sales updates the CTR and routes it to Systems Engineering.

- Traceability: Systems updates the System Requirements Specification (SRS) in the ALM/PLM system and generates an automated impact analysis matrix for Hardware, firmware and Software leads.

- Closed-Loop Verification: R&D engineering leads sign off on technical feasibility and cost impact; updated specifications are re-baselined with customer formal sign-off.

### Loop 2: Part / Mechanical Validation Loop (Mech <-> PCB Design <-> Mfg)

- Mechanism: Mechanical Engineering transfers 3D CAD models of the enclosure and keep-out zones to Hardware PCB layout designers.

- Traceability: Hardware exports 3D STEP files of the populated PCBA back to Mechanical Design for collision and clearance checking.

- Closed-Loop Verification: Joint Mechanical/DFM review generates a signed DFM/DFA clearance certificate before PCB Gerber files are released for tooling.

### Loop 3: Design Verification (DVR) Defect Feedback Loop (Testing <-> Design <-> Quality)

- Mechanism: Functional, Environmental, or EMC failures during DVP testing trigger an automated Critical Defect Ticket in Jira/PLM.

- Traceability: Quality Engineering initiates a preliminary 8D report linked directly to the DVP test case ID and hardware serial number.

- Closed-Loop Verification: Hardware/firmware/Software R&D executes root-cause analysis (5-Why / Ishikawa), issues an Engineering Change Request (ECR), builds modified prototypes, and re-runs the failed DVP test cases to achieve formal closure.

### Loop 4: Post-Release Engineering Change (ECO/ECN) Loop (Quality/Field <-> HW Design <-> SCM <-> Operations)

- Mechanism: Field failures, component obsolescence (EOL), or manufacturing yield improvement initiatives trigger an Engineering Change Request (ECR).

- Traceability: Configuration Management conducts a Cross-Functional Impact Analysis covering cost, inventory scrap, tooling, and re-qualification testing needs.

- Closed-Loop Verification: Upon approval, an Engineering Change Notice (ECN) is issued. ERP system updates eBOM/pBOM, Purchasing updates supplier orders, Production updates Control Plans/SOPs, and Quality submits Re-PPAP or PSW delta documentation to the customer.

# 3. Department-Specific Live Dashboard Specifications

## 1. Sales & Business Development Dashboard

- Metrics Tracked: RFQ Conversion Rate (%), Win/Loss Ratio, OEM Customer Specific Requirement (CSR) Compliance Rate (%), Customer Satisfaction Index (CSAT), Quotation Turnaround Time (Days).

- Target Thresholds: Conversion > 25%, CSR Compliance = 100%, CSAT > 90%, Quote Turnaround < 10 days.

- Data Sources & Refresh Frequency: CRM / ERP System; Daily refresh.

## 2. Embedded Hardware Engineering Dashboard

- Metrics Tracked: Schematics/Layout Milestone Adherence (%), Open DFMEA High AP Action Items, DFM/DFA First-Time Pass Rate (%), Component Obsolescence / Single-Source Risk Count, Engineering Change Orders (ECO) Count per Project.

- Target Thresholds: Schedule Adherence > 95%, High AP Actions Open = 0 at Gate Freeze, DFM Pass > 98%, Single-Source Components < 5% of total BOM.

- Data Sources & Refresh Frequency: PLM / ALM (Jira) System; Real-time / Daily refresh.

## 3. Embedded firmware, software & Cloud Dashboard

- Metrics Tracked: Code Coverage (Unit/Integration Test %), Static Code Analysis Defect Density (MISRA C/C++ violations per KLOC), CI/CD Automated Build Success Rate (%), Sprint Velocity Adherence, Field Over-The-Air (OTA) Deployment Success Rate (%).

- Target Thresholds: Statement Coverage > 95%, Branch Coverage > 90%, Mandatory MISRA Violations = 0, OTA Success Rate > 99.5%.

- Data Sources & Refresh Frequency: Git / Jenkins / SonarQube / Jira; Real-time continuous integration refresh.

## 4. Electronics & Mechanical Manufacturing Dashboard

- Metrics Tracked: First Pass Yield (FPY % for SMT and Final Assembly), Overall Equipment Effectiveness (OEE %), Scrap Rate (PPM), Takt Time Variance (%), Line Unplanned Downtime (Hours).

- Target Thresholds: SMT FPY > 99.2%, Assembly FPY > 98.8%, OEE > 85%, Scrap < 500 PPM, Unplanned Downtime < 1%.

- Data Sources & Refresh Frequency: MES (Manufacturing Execution System) / SCADA; Real-time floor display.

## 5. Quality Assurance & Maintenance Compliance Dashboard

- Metrics Tracked: Customer Field PPM, Internal Scrap PPM, Average 8D Problem Solving Closure Time (Days), Open Audit Non-Conformances (Internal vs External), Calibration & Preventive Maintenance Compliance (%).

- Target Thresholds: Customer PPM = 0, Internal PPM < 300, 8D Closure < 14 days, Open NCs = 0 beyond target dates, Calibration Compliance = 100%.

- Data Sources & Refresh Frequency: QMS Portal / ERP Quality Module; Daily refresh.

## 6. Operations, Accounts & Logistics Dashboard

- Metrics Tracked: On-Time In-Full (OTIF) Delivery (%), Inventory Turnover Ratio, Capacity Utilization Rate (%), Days Sales Outstanding (DSO), Premium Freight Incident Count.

- Target Thresholds: OTIF > 98.5%, Capacity Utilization > 80%, DSO < 45 days, Premium Freight Incidents = 0.

- Data Sources & Refresh Frequency: ERP System (SAP/Oracle); Real-time operational update.

# 4. Department-by-Department IATF 16949 QMS & Audit Framework

The status of QMS documentation artifacts across all operational departments is detailed below.+-----------------------------------------------------------------------------------------------------------------+

|                                     DEPARTMENTAL QMS IMPLEMENTATION STATUS                                     |

+----------------------------+-----------+-------------+---------------+-----------------+--------------+---------+

| Department                 | SOP       | Process KPI | Risk Register | Checklists/Doc  | Skill Matrix | Audit   |

+----------------------------+-----------+-------------+---------------+-----------------+--------------+---------+

| Embedded Hardware          | Done      | Done        | Missing       | Done            | Done         | GAP     |

| Embedded firmware          | Done      | Done        | Done          | Done            | Done         | OK      |

| Software                   | Done      | Done        | Done          | Done            | Done         | OK      |

| Electronics Manufacturing  | Done      | Done        | Missing       | Done            | Done         | GAP     |

| Mechanical Manufacturing   | Done      | Done        | Missing       | Done            | Done         | GAP     |

| Product Delivery / WMS     | Done      | Done        | Done          | Done            | Done         | OK      |

| Service & IT               | Done      | Done        | Done          | Missing         | Done         | GAP     |

| Sales & Marketing          | Done      | Done        | Done          | Done            | Done         | OK      |

| Accounts & Finance         | Done      | Done        | Done          | Done            | Done         | OK      |

| HR & Admin                 | Done      | Done        | Done          | Done            | Done         | OK      |

| Operations                 | Done      | Done        | Done          | Done            | Done         | OK      |

+----------------------------+-----------+-------------+---------------+-----------------+--------------+---------+

## Department Deep-Dive Specifications

### 1. Embedded Hardware Engineering

- SOPs: SOP-HW-01 (Schematic Design & Simulation), SOP-HW-02 (PCB Layout & DFM), SOP-HW-03 (Hardware Verification & DVP Execution).

- Process KPIs: Hardware Design First Time Right (FTR %), Schematic-to-Layout Review Duration, Engineering Change Note (ECN) Frequency per Project.

- Risk Register (5W Analysis):

  - Status: GAP - ACTION REQUIRED.

  - What: Incomplete formal Hardware Risk Register covering component obsolescence and layout thermal bottlenecks.

  - Why/Who/When/Where: Omission identified during internal pre-audit; Assigned to Hardware Lead to publish before IATF Stage 2 Audit.

- Procedural Checklists & Templates: Hardware Peer Design Review Checklist, DFM/DFA Sign-off Template, PCB Component De-rating Worksheet.

- Skill Matrix: Tier 1: Schematic Capture, Circuit Simulation; Tier 2: High-speed PCB Layout, EMI/EMC Troubleshooting; Tier 3: High-frequency RF & Thermal Simulation.

- IATF Audit Evidence Dossier: Clause 8.3 Design Records, DFMEA Records, DVP Verification Reports, ECR/ECN Authorization Binders.

### 2. Embedded firmware & Cloud software if applicable

- SOPs: SOP-FW-01,  (Embedded firmware Architecture & Coding Guidelines - MISRA C), SOP-SW-02 (Embedded firmware Verification & SIL/HIL Testing), SOP-SW-03 (Cybersecurity Implementation & OTA Handling).

- Process KPIs: Code Coverage (%), Defect Density per KLOC, Build Failure Frequency, Sprint Delivery Index.

- Risk Register (5W Analysis): Fully operational. Addresses potential security vulnerabilities, firmware memory overflows, and third-party stack license risks.

- Procedural Checklists & Templates: Code Review Checklist, MISRA Compliance Verification Sheet, HIL Test Case Execution Report Template.

- Skill Matrix: Tier 1: C/C++ Embedded Coding, Unit Testing; Tier 2: AUTOSAR Architecture, HIL Test Setup; Tier 3: Automotive Cybersecurity (ISO 21434) & OTA Cloud Architecture.

- IATF Audit Evidence Dossier: Clause 8.3.3.2 Software Development Capability Records, ASPICE Assessment Reports, Software Release Notes, Static Analysis Logs.

### 3. Software (Enterprise) if  applicable

- SOPs: SOP-SW-01 (Enterprise Software Lifecycle & PLM/ALM Maintenance), SOP-IT-02 (Database Backup & Security Protocol).

- Process KPIs: Enterprise Software Uptime (%), Bug Resolution Velocity, Integration Script Reliability.

- Risk Register (5W Analysis): Fully operational. Focuses on data corruption, PLM sync failures, and API breakages.

- Procedural Checklists & Templates: PLM Migration Audit Checklist, System Access Control Review Template.

- Skill Matrix: Tier 1: SQL/Database Admin; Tier 2: PLM/ERP API Integration; Tier 3: Enterprise Cloud Architecture & ISO 27001 Security.

- IATF Audit Evidence Dossier: System Validation Reports, Backup Restoration Test Logs, Access Rights Audit Trail.

### 4. Electronics Manufacturing (SMT & Assembly)

- SOPs: SOP-EM-01 (SMT Line Setup & Stencil Printing), SOP-EM-02 (Reflow Soldering Profile & Inspection), SOP-EM-03 (Automated Optical Inspection - AOI & ICT Execution).

- Process KPIs: SMT First Pass Yield (FPY %), Component Placement Scrap (PPM), Reflow Oven Temperature Stability Index.

- Risk Register (5W Analysis):

  - Status: GAP - ACTION REQUIRED.

  - What: SMT Process Risk Register missing parameters for moisture-sensitive device (MSD) exposure and ESD control failures.

  - Why/Who/When/Where: Action assigned to SMT Operations Lead for immediate completion.

- Procedural Checklists & Templates: Reflow Profile Log Sheet, MSD Tracking Sheet, SMT Daily Preventive Maintenance Checklist.

- Skill Matrix: Tier 1: SMT Machine Operator, Visual Inspection; Tier 2: Pick-and-Place Programmer, Profile Setup; Tier 3: SMT Line Process Specialist, AOI Algorithm Tuning.

- IATF Audit Evidence Dossier: Clause 8.5 Production Provision, Control Plan Execution Logs, MSD Handling Logs, Maintenance Records.

### 5. Mechanical Manufacturing & Assembly

- SOPs: SOP-MM-01 (Casing Fabrication & CNC Machining), SOP-MM-02 (Automated Screwdriving & Torque Control), SOP-MM-03 (Gland Insertion & Enclosure Sealing Verification).

- Process KPIs: Assembly First Pass Yield (FPY %), Torque Audit Pass Rate (%), Scrap Rate (PPM).

- Risk Register (5W Analysis):

  - Status: GAP - ACTION REQUIRED.

  - What: Mechanical Risk Register lacks risk identification for wire sheath stripping lengths and vendor part segregation.

  - Why/Who/When/Where: Identified via closed-loop quality incident; Risk Register update mandated immediately.

- Procedural Checklists & Templates: Torque Wrench Calibration Checklist, Gland & Harness Sealing Inspection Sheet, Station Setup Checklist.

- Skill Matrix: Tier 1: Assembly Line Operator; Tier 2: Mechanical Jig & Torque Tool Calibration Specialist; Tier 3: Mechanical Manufacturing Quality Engineer.

- IATF Audit Evidence Dossier: Torque Calibration Records, Process Control Sheets, Equipment Maintenance Logs.

### 6. Product Delivery, Warehousing & Logistics

- SOPs: SOP-PD-01 (Finished Goods Receiving & Binning), SOP-PD-02 (Packaging & FIFO Control), SOP-PD-03 (Dispatch Verification & Shipping Documentation).

- Process KPIs: On-Time In-Full (OTIF %), Picking Accuracy Rate (%), Warehouse Space Utilization (%), Shipping Damage Rate (PPM).

- Risk Register (5W Analysis): Fully operational. Focuses on dispatch errors, FIFO breaches, transit damage, and barcode labeling mix-ups.

- Procedural Checklists & Templates: Dispatch Inspection Checklist, Shipping Container Audit Sheet, FIFO Verification Log.

- Skill Matrix: Tier 1: Warehouse Material Handler; Tier 2: ERP WMS System Operator; Tier 3: Logistics & Customs Compliance Specialist.

- IATF Audit Evidence Dossier: Clause 8.5.4 Preservation Records, Packaging Verification Reports, Shipping Audit Trackers, FIFO Verification Logs.

### 7. Service & Enterprise IT

- SOPs: SOP-IT-03 (IT Infrastructure & User Access), SOP-IT-04 (Disaster Recovery & Data Backup), SOP-SRV-01 (Field Return & Warranty Analysis).

- Process KPIs: IT Network Uptime (%), Helpdesk Ticket Resolution Time, Field Warranty Claim Processing Time.

- Risk Register (5W Analysis): Operational. Covers ransomware threats, server room thermal shutdown, and field claim database loss.

- Procedural Checklists & Templates:

  - Status: GAP - ACTION REQUIRED.

  - What: Procedural checklist documentation for server hardware maintenance and field claim intake missing.

  - Why/Who/When/Where: Action assigned to IT & Service Lead to standardize checklists.

- Skill Matrix: Tier 1: IT Helpdesk Support; Tier 2: System Administrator, Network Security; Tier 3: Enterprise Infrastructure & Cyber Resilience Specialist.

- IATF Audit Evidence Dossier: Server Maintenance Logs, Disaster Recovery Mock Drill Reports, Field Claim Analysis Files.

### 8. Sales & Marketing

- SOPs: SOP-SL-01 (RFQ Review & Commercial Quotation), SOP-SL-02 (Customer Contract Review & CSR Identification), SOP-SL-03 (Customer Satisfaction Index Evaluation).

- Process KPIs: Win/Loss Conversion Ratio, Customer CSR Matrix Compliance Rate, Quotation Turnaround Time.

- Risk Register (5W Analysis): Fully operational. Covers price erosion pressures, customer demand volatility, and contractual penalty exposure.

- Procedural Checklists & Templates: RFQ Feasibility Checklist, Customer Contract Review Template, Customer Complaint Logging Sheet.

- Skill Matrix: Tier 1: Commercial Sales Executive; Tier 2: Technical Key Account Manager; Tier 3: Business Development Director / CSR Strategist.

- IATF Audit Evidence Dossier: Clause 8.2 Customer Communication Records, Contract Review Sign-offs, CSR Alignment Matrix, Customer Satisfaction Surveys.

### 9. Accounts & Finance

- SOPs: SOP-AC-01 (Product Costing & CAPEX Approval), SOP-AC-02 (Accounts Payable & Receivable Management), SOP-AC-03 (Financial Risk Control & Asset Auditing).

- Process KPIs: Budget Variance (%), Days Sales Outstanding (DSO), Audit Compliance Index.

- Risk Register (5W Analysis): Operational. Addresses currency exchange volatility, delayed customer payments, and capital allocation bottlenecks.

- Procedural Checklists & Templates: Asset Verification Checklist, Project CAPEX Approval Form, Monthly Financial Audit Checklist.

- Skill Matrix: Tier 1: Billing & Receivables Clerk; Tier 2: Cost Accountant; Tier 3: Financial Controller / Audit Lead.

- IATF Audit Evidence Dossier: Product Costing Sheets, Annual Statutory Financial Audits, Fixed Asset Registers.

### 10. HR & Administrative Services

- SOPs: SOP-HR-01 (Recruitment & Competency Assessment), SOP-HR-02 (Employee Onboarding & Mandatory Training), SOP-HR-03 (Training Effectiveness & Skill Matrix Management).

- Process KPIs: Mandatory Training Completion Rate (%), Competency Index, Staff Turnover Rate (%), Onboarding Cycle Time.

- Risk Register (5W Analysis): Operational. Focuses on skilled labor attrition, training gap escapes to the shop floor, and workplace statutory non-compliance.

- Procedural Checklists & Templates: Operator Qualification Assessment Sheet, Onboarding Verification Checklist, Annual Training Plan Template.

- Skill Matrix: Tier 1: HR Operations Executive; Tier 2: Technical Trainer & Competency Auditor; Tier 3: HR Director & Organizational Design Specialist.

- IATF Audit Evidence Dossier: Clause 7.2 & 7.3 Competency and Awareness Records, Individual Skill Evaluation Sheets, Training Attendance & Verification Logs.

### 11. Operations & Program Management Office (PMO)

- SOPs: SOP-OP-01 (APQP Gate Management & Project Tracking), SOP-OP-02 (Capacity Planning & S&OP Execution), SOP-OP-03 (Engineering Change Management Oversight).

- Process KPIs: On-Time APQP Gate Pass Rate (%), S&OP Schedule Adherence (%), Overall Equipment Effectiveness (OEE).

- Risk Register (5W Analysis): Fully operational. Covers multi-project resource contention, customer SOP date delays, and supply chain bottlenecks.

- Procedural Checklists & Templates: APQP Gate Sign-off Checklists (Gate 1 to 5), Program Status Report Template, Capacity Audit Worksheet.

- Skill Matrix: Tier 1: Project Planner; Tier 2: Program Manager (PMP/APQP Certified); Tier 3: Director of Program Operations.

- IATF Audit Evidence Dossier: Clause 8.1 Operational Planning Records, Gate Sign-off Certificates, Executive Project Review Minutes.

## Quality Incident & Closed-Loop Problem Solving Case Study

To satisfy IATF 16949 Clause 10.2 (Non-conformity and Corrective Action), the QMS incorporates documented lessons learned from actual manufacturing non-conformances.+-----------------------------------------------------------------------------------------------------------------+

|                              CASE STUDY: CLOSED-LOOP 8D CORRECTIVE ACTION REPORT                                |

+-----------------------+-----------------------------------------------------------------------------------------+

| Incident ID           | NC-2026-TCU03-001 (Customer Complaint from ITL / Hoshiarpur)                   |

| Affected Part         | TCU-03 / Part Number: SSMIND001 (Mfg Date: 30/10/2025)                        |

| Problem Description   | Core wire visible outside the gland due to excessive harness sheath length.   |

| Cross-Functional Team | (Sr. Mgr Elec Mfg), (Mech Mfg Quality), (Elec Quality),  |

|                       | (Sr. Mgr Elec Mfg), (Sr. Eng Elec Mfg), (Eng Elec Mfg),  |

|                       | (Head Mech Mfg).                                                    |

+-----------------------+-----------------------------------------------------------------------------------------+

| Technical Root Cause  | Occurrence: Absence of designated Red Bins at supplier (Yubiti Automobile) sheath     |

|                       | setting workstation caused mixing of OK and NG parts. Un-trained new joiner    |

|                       | operator handled part segregation without adequate skill verification.        |

|                       | Detection: Final Quality Check (FQC) checklist lacked 100% check for sheath length.|

+-----------------------+-----------------------------------------------------------------------------------------+

| Corrective Actions    | 1. Supplier deployed Green Bins for OK parts and Red Bins for Rejection parts.  |

|                       | 2. Operator training completed and Skill Matrix records updated.              |

|                       | 3. Updated FQC Checksheet to mandate 100% gland core wire inspection.         |

+-----------------------+-----------------------------------------------------------------------------------------+

# 5. Master Organization Governance, Risk, Facility & Statutory Audit Dossier

## 1. Quality, Safety & Environmental Certifications

+-------------------------------------------------------------------------------------------------------------------+

|                                  ORGANIZATIONAL CERTIFICATIONS & STATUTORY DIRECTIVE                              |

+--------------------------+-----------------------+-----------------------+------------------+---------------------+

| Standard / License       | Issuing Authority     | Certificate Number    | Valid Until      | Audit Cycle         |

+--------------------------+-----------------------+-----------------------+------------------+---------------------+

| IATF 16949:2016          | TÜV NORD / DNV        | 44 111 200543         | 14.11.2027       | Annual Recert       |

| ISO 9001:2015            | TÜV NORD / DNV        | 04 100 200543         | 14.11.2027       | Integrated Audit    |

| ISO 27001:2022           | BSI / Intertek        | IS 789012             | 22.08.2028       | Surveillance Audit  |

| ISO 14001:2015           | Bureau Veritas        | IND.24.5678/ENV       | 10.03.2027       | Annual Recert       |

| ISO 45001:2018           | Bureau Veritas        | IND.24.5678/HS        | 10.03.2027       | Annual Recert       |

| Factory License          | Chief Inspector Factories| FACT-UP/NOD/2024/44   | 31.12.2026       | Annual Renewal      |

| UPPCB CTO (Air & Water)  | UP Pollution Control  | CTO/UPPCB/2025/9981   | 31.03.2028       | Renewal Baseline    |

| Fire Safety NOC          | State Fire Service    | NOC-FIRE/2026/112     | 15.01.2027       | Annual Inspection   |

| DG Set Env Clearance     | Central Pollution Control| CPCB-DG-CERT-4412    | Lifetime Valid   | Periodic Emission   |

+--------------------------+-----------------------+-----------------------+------------------+---------------------+

## 2. Corporate Risk Management Register

+-----------------------------------------------------------------------------------------------------------------------+

|                                              CORPORATE RISK MATRIX                                                    |

+------------+-----------------------------------+------+------+------+-------------------------------------------------+

| Risk ID    | Risk Category & Threat Description| Prob | Imp  | Risk | Mitigation Strategy & Safeguards                |

+------------+-----------------------------------+------+------+------+-------------------------------------------------+

| R-FIR-01   | Fire in Reflow Oven / SMT Area    | Low  | High | Med  | Automated FM200 Fire Suppression, Thermal Cams  |

| R-FIR-02   | Electrical Short Circuit in Lab   | Med  | High | High | Thermography Audits, Class C Extinguishers      |

| R-NAT-01   | Facility Flooding during Monsoon  | Low  | High | Med  | Elevated Floor (1.2m), Dual Sump Pumps          |

| R-NAT-02   | Grid Power Loss due to Storms    | High | Med  | Med  | N+1 Redundant DG Sets, Uninterrupted UPS        |

| R-ALT-01   | Single-Source Microcontroller EOL | Med  | High | High | Dual-Sourcing Policy, Pin-to-Pin Alt IC Pre-qual|

| R-ALT-02   | Raw Material SMT Paste Shortage   | Med  | Med  | Med  | Safety Stock Buffer (6 Weeks) in Cold Storage   |

| R-SEC-01   | Firmware IP Source Code Theft     | Low  | High | Med  | DLP Agent, Air-Gapped Repos, ISO 27001 Controls |

| R-SEC-02   | Ransomware Attack on ERP/MES      | Low  | High | High | Daily Offsite Backups, Zero-Trust Architecture   |

| R-ENV-01   | Hazardous Chemical / Solder Leak  | Low  | High | Med  | Secondary Containment Trays, E-Waste Tie-ups    |

| R-ENV-02   | ETP / STP Effluent Water Breach   | Low  | High | Med  | Zero Liquid Discharge (ZLD) Monitoring System   |

+------------+-----------------------------------+------+------+------+-------------------------------------------------+

## 3. Management Review Protocol (IATF Clause 9.3)

- Frequency: Bi-Annual (Held every 6 months in April and October).

- Mandatory Inputs: Internal/external audit results, customer feedback/CSAT, process performance & product conformity (FPY/PPM), status of preventive/corrective actions (8Ds), follow-up actions from prior management reviews, changes in external context/risks, resource adequacy, and effectiveness of supplier performance monitoring.

- Mandatory Outputs: Decisions on quality improvement opportunities, resource/CAPEX approvals, QMS updates, and product baseline changes. Minutes and action tracking matrices are retained for a minimum of 10 years.

## 4. Internal Audit Schedule (IATF Clause 9.2)

- QMS System Audit: Conducted annually across all 11 departments using IATF 16949 audit checklists.

- Manufacturing Process Audit (IATF 9.2.2.3): Conducted every 6 months for each manufacturing process (SMT Line, Mechanical Assembly, Final Test) covering all shifts (CQI-17 / VDA 6.3 aligned).

- Product Audit (IATF 9.2.2.4): Conducted monthly on packaged finished goods prior to dispatch to verify dimensional, functional, packaging, and barcode compliance.

## 5. Environmental, Health & Workplace Safety (EHS) Framework

- EHS Policy Compliance: Mandatory usage of ESD Personal Protective Equipment (PPE), safety eyewear, and anti-static shoes across production lines.

- Waste Management Protocol: Hazardous waste (solder dross, chemical wipes, spent solvent) stored in designated hazardous waste rooms and handed over to state-authorized recyclers every 90 days.

- Emergency Preparedness: Quarterly fire drills, monthly emergency exit door audits, bi-weekly eye-wash station flushing tests, and annual structural stability certification.